- OLT CLI upgrade guide

Step1: Connect PC to OLT AUX port.

OLT AUX port default management ip 192.168.8.100.

Config PC IP 192.168.8.101 255.255.255.0

Step2:Running TFTP server tool. Select the firmware files directory.

Step3:Upgrade OLT firmware(img and ubi or bin files)

Login: admin

Password: admin

epon-olt> enable

Password: admin

epon-olt# config terminal

epon-olt(config)# download tftp image V1600D_IMG_V2.03.20.img 192.168.8.101

epon-olt(config)# download tftp image V1600D_UBI_V2.03.20.ubi 192.168.8.101

or in some old olt firmware download like this:

epon-olt# config terminal

epon-olt(config)# debug mode

epon-olt(debug-mode)# download tftp image V1600D_IMG_V2.03.20.img 192.168.8.101

epon-olt(debug-mode)# download tftp image V1600D_UBI_V2.03.20.ubi 192.168.8.101

for example:

epon-olt(config)# download tftp image V1600D_IMG_V2.03.20.img 192.168.8.101

Trying to download V1600D_IMG_V2.03.20.img from server 192.168.8.101, please wait...

2000/01/01 22:57:07   Download File Success    download V1600D_IMG_V2.03.20.img success

Image file V1600D_IMG_V2.03.20.img is kernel

Erase flash mtd /dev/mtd2

Trying to save to flash,len = 0x5b82c4, mtd size 0xd00000 please wait...

................................................................

................................................................

................................................................

................................................................

................................................................

..............................................

Save to flash success...

2000/01/01 22:57:10   Upgrade File Success     upgrade V1600D_IMG_V2.03.20.img success

epon-olt(config)# download tftp image V1600D_UBI_V2.03.20.ubi 192.168.8.101

Trying to download V1600D_UBI_V2.03.20.ubi from server 192.168.8.101, please wait...

2000/01/01 22:58:43   Download File Success    download V1600D_UBI_V2.03.20.ubi success

Image file V1600D_UBI_V2.03.20.ubi is rootfs

Erase flash mtd /dev/mtd4

Trying to save to flash,len = 0x27e0000, mtd size 0x4000000 please wait...

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

................................................................

........................................................

Save to flash success...

2000/01/01 22:58:57   Upgrade File Success     upgrade V1600D_UBI_V2.03.20.ubi success

Step4: save configuration (if necessary) and reboot olt.

epon-olt(config)# copy running-config startup-config

2000/01/01 23:01:52   System Config Save       save config by command

Configuration saved to /mnt/custfs/usrcfg.conf

epon-olt(config)# reboot

Are you sure want to reboot system? [Y/N]y